| 1. IDENTIFICATION OF THE SUBSTANCE AND OF THE COMPANY |  |
| --- | --- |
| 1.1 Product identifier |  |
| Commercial Product Name: | COAL TAR PITCH ELECTRODE |
| Chemical Name | Pitch, coal tar, high-temp. |
| CAS#: | 65996-93-2 |
| EC#: | 266-028-2 |
| REACH Registration Number | 01-2119541809-29-0023 |
| 1.2 Relevant identified uses of the substance and uses advised against |  |
| Identified uses: | Manufacture of anode mass, coal and graphite products, structural coal-graphite materials, electro-coal produce. Intermediate use only. PROC 1, PROC 2, PROC 3, PROC 8a, PROC 8b, PROC 15. ERC6a. |
| Uses advised against: | None specified. |
| 1.3 Details of the supplier of the safety data sheet |  |
| Only representative | Metinvest International Italia s.r.l. Via Antonio Salieri 36, 37050, Vallese di Oppeano (VR) Italy Telephone: +39 045 6983398; +39 3482235002 Email: REACHSDS@metinvestholding.com |
| Manufacturer/Supplier | Private Joint Stock Company «KAMET-STEEL» 51925, Ukraine Dnipropetrovsk oblast, Kamianske, 18b Soborna str Ident Code 05393085 Telephone: +38 (0569) 58-91-49 Email: office.kamet-steel@metinvestholding.com |
| National contact | Onishchenko Vitaliy Email: vitaliy.onishchenko@metinvestholding.com |
| 1.4 Emergency telephone number |  |
| 112 Is available 24/7 |  |

| 2. HAZARDS IDENTIFICATION |  |  |
| --- | --- | --- |
| 2.1 Classification of the substance |  |  |
| Classification according to Regulation (EC) № 1272/2008 (CLP) |  | Additional information |
| Skin Sens. 1; H317. Muta. 1B; H340. Carc. 1A; H350. Repr. 1B; H360FD. Aquatic Acute 1; H400; M=1000. Aquatic Chronic 1; H410; M=1000. | For full text of H-phrases see section 2 and 16 of the SDS |
| 2.2 Label elements |  |  |
| Identifier | Index №: 648-055-00-5 |  |
| Hazard pictograms |  |  |
| Signal word | Danger |  |
| Hazard statements | H317: May cause an allergic skin reaction. H340: May cause genetic defects. H350 : May cause cancer. H360FD: May damage fertility. May damage the unborn child. H400: Very toxic to aquatic life. H410: Very toxic to aquatic life with long lasting effects. |  |
| Precautionary statements | P201: Obtain special instructions before use. P202: Do not handle until all safety precautions have been read and understood. P280: Wear protective gloves/protective clothing/eye protection/face protection. P308 + P313: IF exposed or concerned: Get medical advice/attention. P261: Avoid breathing dust/fume/gas/mist/vapours/spray. P273: Avoid release to the environment. |  |
| Additional information |  |  |
| 2.3 Other hazards |  |  |
| The substance meets the criteria for PBT or vPvB |  |  |

| 3. COMPOSITION/INFORMATION ON INGREDIENTS |  |  |  |
| --- | --- | --- | --- |
| 3.1 Substances |  |  |  |
| Chemical name | ЕС # | CAS # | Concentration, range % |
| Pitch, coal tar, high-temp. | 266-028-2 | 65996-93-2 | 100% |

| 4. FIRST AID MEASURES |  |
| --- | --- |
| 4.1 Description of first aid measures |  |
| General information | In case of inhalation: Move person to fresh air. Remove contaminated clothing and shoes. Keep patient warm and at rest preferably in a comfortable upright sitting position. If breathing stops, provide artificial respiration. Get medical attention if any discomfort continues. In case of eye contact: Immediately flush with plenty of water for up to 15 minutes. Remove any contact lenses and open eyes wide apart. Get medical attention immediately. Continue to rinse. In case of ingestion: Never make an unconscious person vomit or drink fluids! Do not induce vomiting! If vomiting occurs, keep head low so that stomach content doesn't get into the lungs. Rinse mouth thoroughly. Get medical attention immediately! In case of skin contact: Remove contaminated clothing and shoes. Wash off immediately with soap and plenty of water while removing all contaminated clothes and shoes. In case of burns apply cold water until pain subsides then seek medical advice. Call an ambulance immediately. Continue flushing during transport to hospital. |
| 4.2 Most important symptoms and effects, both acute and delayed |  |
| In case of inhalation | Acute respiratory effects caused by overexposure to coal tar may include coughing, sneezing, and swollen or irritated nasal mucosa and sinuses. |
| In case of eye contact | Vapors or mist or direct exposure may cause irritation to the eyes and mucous membranes. |
| In case of skin contact | May cause skin irritation characterized by skin itching, burning, swelling and redness. Short-term exposures may also cause transient photosensitization. |
| In case of ingestion | Unlikely route of exposure. If ingested, may cause headache, drunkenness, nausea, vomiting, weakness, convulsions, unconsciousness and coma. Aspiration of this material into the lungs can cause chemical pneumonia. |
| Information to physician | Treat symptomatically and supportively. First aid personnel must be aware of own risk during rescue. Repeated excessive exposures may cause blood disorders such as anemia and leukemia. Repeated excessive exposures may cause liver and/or kidney effects or damage. Material has been related to cancer in humans. |
| First aid arsenal | Universal medical kit with a set of drugs (in consultation with the medical department of the enterprise). |
| 4.3 Indication of any immediate medical attention and special treatment needed |  |
| Immediate medical attention is expected. |  |

| 5. FIREFIGHTING MEASURES |  |
| --- | --- |
| 5.1 Extinguishing media |  |
| Flammable properties | CTP(ht) is not flammable, not explosive and not oxidising |
| Suitable extinguishing media | Use extinguishing measures that are appropriate to local circumstances and the surrounding environment. Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. |
| Unsuitable extinguishing media | Do not use water jet as an extinguisher, as this will spread the fire. |
| 5.2 Special hazards arising from the substance |  |
| Hazardous combustion products | Carbon oxides and other toxic gases or vapours. Fire or high temperatures creates toxic fumes of Carbon monoxide (CO), Carbon dioxide (CO2), Nitrous gases (NOx). Sulphurous gases (SOx). |
| Special protective equipment for fire-fighters | Self contained breathing apparatus and full protective clothing must be worn in case of fire. |
| 5.3 Advice for firefighters |  |
| Keep up-wind to avoid fumes. Move container from fire area if it can be done without risk. Use water SPRAY only to cool containers! Do not put water on leaked material. Keep run-off water out of sewers and water sources. Dike for water control. |  |

| 6. ACCIDENTAL RELEASE MEASURES |  |
| --- | --- |
| 6.1. Personal precautions, protective equipment and emergency procedures |  |
| 6.1.1. For non-emergency personnel | Evacuate non-emergency personel from contaminated area. Only qualified personnel equipped with suitable PPE may intervene. Avoid inhalation of vapours and contact with skin and eyes. Provide adequate ventilation. |
| 6.1.2. For emergency responders | Use personal protection: tightly closed chemical protection suit and a self-contained breathing apparatus. |
| 6.2 Environmental precautions |  |
| Do not allow to enter drains, sewers or watercourses. The product should not be dumped in nature but collected and delivered according to agreement with the local authorities. |  |
| 6.3 Methods and material for containment and cleaning up |  |
| Stop leak if possible without risk. Prevent spreading over a wide area (e.g. by containment or oil barriers). Do not touch spilled material! If material is at an elevated temperature, use sand or earth to make a barrier whilst material solidifies. Allow material to go solid and collect spillage with shovel, broom or the like. Transfer to a container for disposal. Containers with collected spillage must be properly labelled with correct contents and hazard symbol. Clean contaminated area with oil-removing material. |  |
| 6.4 Reference to other section |  |
| Information about personal precautions - see Section 8. Information about waste disposal - see Section 13. |  |

| 7. HANDLING AND STORAGE |  |
| --- | --- |
| 7.1 Precautions for safe handling |  |
| Precautions for safe handling | Avoid contact with skin and eyes. Avoid breathing vapours. Avoid release to the environment. Persons with a history of skin sensitization problems or pregnant or breastfeeding women should not be employed in any process in which this product is used. Wear protective personal protection as stated in Section 8. |
| Fire preventions | Use explosion-proof electrical (ventilating, lighting and material handling) equipment. Use of non-sparking tools is preferable. |
| Aerosol and dust generation preventions | Handle only in a place equipped with local exhaust (or other appropriate exhaust) ventilation. In case of insufficient ventilation, wear suitable respiratory equipment. Keep the concentration of product’s components in air lower OELs. |
| Electrostatics prevention | Ground containers, tanks and transfer or receiving equipment. |
| Safe transporting | Do not violate the integrity of container. During loading works, execute instructions and rules for the appropriate works. For further information see section 14. |
| Advice on general occupational hygiene | Wash hands thoroughly after handling. Ensure that a high level of personal hygiene is maintained when using this product. Avoid eating, drinking and smoking in the working area. Wash hands after using the product. Change contaminated clothes at the end of working shift. Regular cleaning of process equipment and surrounding areas is recommended. |
| 7.2 Conditions for safe storage, including any incompatibilities |  |
| Technical measures and storage conditions | Do not enter storage areas and confined spaces unless adequately ventilated. Empty containers retain product residue and can be hazardous. Do not reuse container. |
| Packaging materials | Molten CPT is stored in closed containers heated by deaf steam at a temperature exceeding the softening temperature of the pitch by no more than 90°C, avoiding local overheating. |
| Incompatible materials | Oxidising materials |
| Requirements for storage rooms and vessels | Store locked up. Keep container tightly closed and sealed until ready for use. Containers that have been opened must be carefully resealed and kept upright to prevent leakage. Provide appropriate containment to avoid environmental contamination. |
| 7.3 Specific end use(s) |  |
| None. |  |

| 8. EXPOSURE CONTROLS/PERSONAL PROTECTION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.1 Control parameters |  |  |  |  |  |  |  |  |  |
| Occupational exposure limits: |  |  |  |  |  |  |  |  |  |
| Limit value type (country of origin) | Substance name |  | CAS# |  | Monitoring procedures | Occupational exposure limit value |  |  |  |
|  |  |  |  |  |  | Long term mg/m3 |  | Short term mg/m3 |  |
| Belgium (VLEP) | Coal tar pitch volatiles (PAHs) as benzene soluble fraction |  | - |  | Area Air Sampling | 0,2 |  | - |  |
| Denmark (OEL) |  |  |  |  |  | 0,2 |  | - |  |
| France (VLE) |  |  |  |  |  | 0,2 |  | - |  |
| Spain (VLA) |  |  |  |  |  | 0,2 |  | - |  |
| DNEL/DMEL or AEL/AEC values: |  |  |  |  |  |  |  |  |  |
| Substance name | Worker |  |  | Consumer |  |  | Exposure route |  | Exposure frequency |
| Pitch, coal tar, high-temp. | DNEL = 0,002 mg/m³ |  |  | DNEL = 0 mg/m³ |  |  | inhalation |  | Long-term |
|  | DNEL = 0,2 mg/kg bw/day |  |  | - |  |  | dermal |  | Long-term |
|  | - |  |  | DNEL = 0 mg/m³ |  |  | oral |  | Long-term |
| PNEC values: |  |  |  |  |  |  |  |  |  |
| Substance name | Environmental compartment |  |  | Value |  |  | AF |  | Remarks |
| Pitch, coal tar, high-temp. | aqua (freshwater) |  |  | PNEC = 0 mg/L |  |  | 10 |  | - |
|  | aqua (marine water) |  |  | PNEC = 0 mg/L |  |  | 10 |  | - |
|  | sediment (freshwater) |  |  | PNEC = 0,84 mg/kg sediment dw |  |  | - |  | - |
|  | sediment (marine water) |  |  | PNEC = 0,366 mg/kg sediment dw |  |  | - |  | - |
|  | soil |  |  | PNEC = 0,01 mg/kg soil dw |  |  | 50 |  | - |
|  | STP |  |  | PNEC = 3,6 mg/L |  |  | 10 |  | - |
| Biological limit values: |  |  |  |  |  |  |  |  |  |
| EH40 Biological monitoring guidance value, Polycyclic aromatic hydrocarbons (PAHs) : 4 μmol 1-hydroxypyrene/mol creatinine in urine. |  |  |  |  |  |  |  |  |  |
| 8.2 Exposure controls |  |  |  |  |  |  |  |  |  |
| 8.2.1. Appropriate engineering controls |  |  |  |  |  |  |  |  |  |
| Provide adequate general and local exhaust ventilation. All handling to take place in well-ventilated area. Ensure that eye flushing systems and safety showers are located close to the working place. |  |  |  |  |  |  |  |  |  |
| 8.2.2. Individual protection measures, such as personal protective equipment |  |  |  |  |  |  |  |  |  |
| Respiratory protection |  | In case of inadequate ventilation or risk of inhalation of vapours, use suitable respiratory equipment with combination filter (type A2/P3). |  |  |  |  |  |  |  |
| Eye/face protection |  | Wear safety googles, full-face visor or shield. |  |  |  |  |  |  |  |
| Skin/body protection |  | Wear protective gloves made of nitrile or butyl rubber (0.35mm) and in the case of use of molten form wear appropriate heat resistant gloves. Discard gloves as soon as any signs of degradation are noticed. Wear apron or protective clothing. Refer to EN 1149. |  |  |  |  |  |  |  |
| General hygiene considerations |  | Use good personal hygiene practices. Wash hands before eating, drinking, smoking, or using toilet facilities. |  |  |  |  |  |  |  |
| 8.2.3. Environmental exposure controls |  |  |  |  |  |  |  |  |  |
| Measures to prevent exposure |  | Do not allow product to contaminate waters, sewers and soil. Emissions from ventilation or work process equipment should be checked to ensure they comply with the requirements of environmental protection legislation. In some cases, fume scrubbers, filters or engineering modifications to the process equipment may be necessary to reduce emissions to acceptable levels. |  |  |  |  |  |  |  |

| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |
| --- | --- |
| 9.1 Information on basic physical and chemical properties |  |
| Appearance | Liquid of black color (molten product). |
| Odour | Aromatic. |
| Odour threshold (mg/m3) | Not available |
| pH | Not available |
| Melting point/range (°C) | 67-150°C softening range |
| Initial boiling point/range (°C) | > 360 °C |
| Flash point (°C) | >210°C |
| Auto-ignition temperature (°C) | > 560 °C |
| Decomposition temperature (°C) | > 400 °C |
| Evaporation rate | Not available |
| Flammability | Non flammable |
| Lower-Upper flammability or explosive limits | Not applicable |
| Vapour pressure (kPa at 20°C) | < 0.1 |
| Vapour density (g/L) | Not applicable |
| Relative density | 1.15 – 1.40 |
| Water solubility (20°C in g/l) | ~ 0.040 |
| Partition coefficient n-Octanol/Water (log Po/w) | Not applicable |
| Viscosity (mPa s) | 230 |
| Explosive properties | Not explosive |
| Oxidizing properties | Not oxidizing |
| 9.2 Other information |  |
| Water solubility: the total cumulative amount of water-extractable EPA PAHs amounted to approx. 370 µg/10g (= ~0.004 %). The water-soluble fraction of single PAHs remained far below their theoretical water-solubilities, which confirms that freely available PAHs got gradually exhausted. Different PAHs have wide range of water solubility values from 31900 to 0.1 µg/l. |  |

| 10. STABILITY AND REACTIVITY |  |
| --- | --- |
| 10.1 Reactivity | No specific reactivity hazards associated with this product. |
| 10.2 Chemical stability | The product is stable upon appropriate handling and storage conditions. |
| 10.3 Possibility of hazardous reactions | Polymerisation can occur at high temperatures. |
| 10.4 Conditions to avoid | Avoid excessive heat, open flame, ignition sources, and incompatible materials. |
| 10.5 Incompatible materials | Oxidizing agents. |
| 10.6 Hazardous decomposition products | Carbon oxides and other toxic gases or vapours. Fire or high temperatures creates toxic fumes of Carbon monoxide (CO), Carbon dioxide (CO2), Nitrous gases (NOx). |

| 11. TOXICOLOGICAL INFORMATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11.1 Information on toxicological effects. |  |  |  |  |  |
| Toxicokinetics, metabolism and distribution |  |  |  |  |  |
| PAHs are absorbed through the pulmonary tract, the gastrointestinal tract, and the skin with different rate of absorption. Absorption from the gastrointestinal tract occurs rapidly in rodents, but metabolites return to the intestine via biliary excretion. PAH mixtures reach the lungs, where they become bound to DNA. PAH are widely distributed throughout the organism after administration by any route and are found in almost all internal organs, but particularly those rich in lipids. PAHs can cross the placental barrier and reach fetal tissues. The metabolism of PAH to more water-soluble derivatives is complex. In general, parent compounds are converted into intermediate epoxides following reaction catalysed by cytochrome P450-dependent mono-oxygenases, which are further transformed by rearrangement or hydration to yield phenols or diols and following secondary oxidation - to yield tetrols, which can themselves be conjugated with sulfuric or glucuronic acids or with glutathione. Most metabolism results in detoxification, but some PAH are activated to DNA-binding species, principally diol epoxides, which can initiate tumours. PAH metabolites and their conjugates are excreted via the urine and faeces, but conjugates excreted in the bile can be hydrolysed by enzymes of the gut flora and reabsorbed. |  |  |  |  |  |
| Acute toxicity: |  |  |  |  |  |
| Substance name | Exposure | Value | Exposure time period | Species | Method (as is, equivalent or similar) |
| Pitch, coal tar, high-temp. | oral | LD50 > 15000 mg/kg bw | single dose | rat | OECD Guideline 401 |
|  | dermal | LD50 > 2 000 mg/kg bw | 24 hours | rat | OECD Guideline 402 |
| Irritation | Skin | CTP is not classified as corrosive or irritative for skin. But among workers exposed to CTP (fumes) dermal effects including pitch warts (clinical and histological keratoacathomas), tar keratoses, acne (morphologically distinguishable from those seen in chloracne) and folliculitis of the thighs and forearms were observed. |  |  |  |
|  | Eyes | CTP is not classified as causing eye damage or irritation. But still human data on occupation exposure to CTP (fumes, volatiles and dust, not further specified) show eye irritation and, after repeated exposure, chemosis of the conjunctiva, ulceration and infiltration of the cornea, deep staining of the cornea, and conjunctival discoulouration and irritation. |  |  |  |
|  | Remarks | None |  |  |  |
| Respiratory or skin sensitization |  | CTP is classified as Skin sensitizier Category 1 since CTP(ht) may contain up to 1.5% BaP. |  |  |  |
| Germ cell mutagenicity |  | CTP is classified as mutagen Category 1B. From mutagenicity testing in S. typhimurium conducted according to OECD guidelines 471, it is concluded that CTP is a bacterial mutagen. CTPHT contains a variable amount of mutagenic PAHs includeing benzo[a]pyrene with amongst others like BaP with hazard classification Muta 1B; H340. This mainly leads to product classification. |  |  |  |
| Carcinogenicity |  | CTP is classified as carcinogen Category 1B. Oral studies with coal tar in mice resulted in increased tumour incidences in various organs, including the liver, lung, and forestomach. Oral studies with BaP resulted in increased tumour incidences in amongst others. The liver, forestomach, and epidermal structures in rats and the forestomach and the upper digestive tract in mice. Inhalation of CTP(ht) caused broncho-alveolar lesions and lung tumours in rats and mice, while dermal exposure to CTP(ht) caused skin tumours in mice. Experimental values are as follows: LOAEL (Carc.) = 120 mg/kg/d (mouse) (OECD guideline 451) NOAEL (Carc.) = 36 mg/kg/d (mouse) (OECD guideline 451) Furthermore, it is noted that lung cancer has a larger impact than bladder cancer on the excess life time risks and is therefore the most critical type of cancer with regard to exposure to CTP(ht). |  |  |  |
| Toxicity for reproduction |  | CTP is classified as toxic for reproduction system Category 1B. No experimental data on the potential reproduction toxicity of CTPHT were available. High-boiling coal liquid, coal tar derived products, and creosote have been shown to produce reproductive toxicity in animals by the inhalation, oral, and dermal routes. High-boiling coal liquid had effects on fertility in a repeated dose inhalation toxicity study (13weeks): NOAEC = 30 mg/m3. At the highest tested concentration also decreased ovary weights and loss of luteal tissue were observed. Coal tar derived products and coal tar creosote had no effects on fertility in mouse studies with NOAELs of 344 mg/kgbw/day and 100 mg/kg, respectively. In a summary of a multigeneration study it is reported that creosote had effects on fertility in rats (at a dose level of 25 g/kgbw/day) below maternal toxic doses (75 mg/kgbw/day). |  |  |  |
| STOT-SE, STOT-RE |  | The substance does not possess specific target organ toxicity following either single or repeated exposure. |  |  |  |
| Repeated dose toxicity: |  |  |  |  |  |
| Regarding animals, since apart from one oral study of limited significance in pigs, no repeated dose toxicity studies with CTP(ht) addressing effects other than carcinogenicity were available and no NOAEL for non-carcinogenic effects could be derived from these studies. In humans no statistical significant effects on lung function parameters were found in a group of phosphorus rock refinery workers exposed at the time of study to about 0.1 mg/m3 CTPV in addition to other substances. |  |  |  |  |  |

| 12. ECOLOGICAL INFORMATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 12.1 Toxicity: |  |  |  |  |  |
| Aquatic toxicity: |  |  |  |  |  |
| Chemical name | Aquatic toxicity | Effect dose | Exposure time | Species | Method (as is, equivalent or similar) |
| Pitch, coal tar, high-temp. | Acute toxicity to fish | LL50 = 100 - 1000 mg/L | 96 hours | Oryzias latipes | OECD Guideline 203 |
|  | Long-term toxicity to fish | NOEC = 4 µg/L | 42 days | Danio rerio | OECD Guideline 210 |
|  | Acute toxicity to aquatic invertebrates | EC50 = 59.7 µg/L | 24 hours | Daphnia magna | OECD Guideline 202 |
|  | Long term-toxicity to aquatic invertebrates | EC10 = 0.503 µg/L | 7 days | Ceriodaphnia dubia | AFNOR |
|  | Toxicity to aquatic algae and cyanobacteria | EC10 = 0.78 µg/L | 72 hours | Pseudokirchnerella subcapitata | Norme Francais EN 28692 |
|  | Toxicity to microorganisms | EL0 = 10 000 mg/L | 17 hours | Pseudomonas putida | DIN38412, part L8 |
| An alternative approach for the environmental classification of CTPHT was taken, considering CTPHT as a ‘mixture’. The 16 individual EPA-PAHs were analysed with respect to their acute aquatic effects data and the lowest available EC50 or LC50 was chosen as a point of departure for acute aquatic hazard classification. These lowest acute toxicity data were combined with degradability and bioaccumulation data to come to a classification for each individual PAH. Since all the PAHs to which an M-factor could be assigned are classified as Aquatic Acute 1, all their contributions were summed to come to an overall contribution (in %) to the toxicity of CTPHT, i.e. 14521%. Since this value is (far) above the 25% limit CTPHT is classified as Aquatic Acute 1 according to CLP Regulation. All individual PAHs are classified as Category Aquatic Chronic 1 (H410), with the exception of benzo[b]fluoranthene Based on these toxicity data and a typical content of the different PAHs, CPTHT should be classified as Category Aquatic Chronic 1 (H410), regardless of the species group considered (total percentages are all > 25%). RAC of ECHA agreed to suggest harmonised M-factors of 1000 for both categories Aquatic Acute and Aquatic Chronic, based on acute and chronic toxicity data of individual PAHs and the typical PAH composition of binder pitch. |  |  |  |  |  |
| 12.2 Persistence and degradability |  |  |  |  |  |
| Abiotic Degradation |  |  |  |  |  |
| PAHs are oxidised by the OH- and NO3-radicals and O3 in the atmosphere. Lifetime of 2- to 4-ring PAHs ranges according to available data from a few hours to several days according to rate constants which correspond to atmospheric degradation half-lives This indirect photolysis occurs mainly for PAHs in gas phase. Larger PAHs are present in the atmosphere adsorbed to fine particles which reduces degradation rate considerably. Direct phototransformation rates for particle bound PAHs are depending on the carbon content of the substrate. Life-times of one hour up to eight days for 4- to 6-ring PAHs adsorbed to particles have been experimentally determined. PAHs are stable against hydrolysis but they are subject to direct and indirect photochemical transformation also in water. Conditions supporting phototransformation exist up to a few centimetres depth below the water surface. As relevant exposure occurs in the whole water column and, in case of PAHs, especially in sediment and soil, photodegradation is not considered to have impact on the persistency in the environment. |  |  |  |  |  |
| Biodegradation |  |  |  |  |  |
| PAHs up to four rings have been observed to biodegrade in aquatic screening tests in aerobic conditions and can be considered inherently biodegradable. Larger PAHs are biodegraded very slowly. Anaerobic degradation is considered negligible for PAHs although some evidence on anaerobic transformation is available. Hence, PAHs are expected to be persistent in the anaerobic layer of marine sediments. Many microbial species present in soil are known to biodegrade PAHs. Suggested bidegradation half-lifes of PAHs in various environmental compartments vary from 4 till 120 days in water, 100-1000 days in soil and 400- 30000 days in sediment. |  |  |  |  |  |
| 12.3 Bioaccumulative potential |  |  |  |  |  |
| The variable composition of CTPHT hampers a solid conclusion on the bioaccumulation of this compound. BCF values for the 16 EPA-PAHs, which are all found in CTPHT, are in many cases above 5000, thus fulfilling the very-bioaccumulative criterion. For CPTHT it appears therefore reasonable to classify this compound as vB. |  |  |  |  |  |
| 12.4 Mobility in soil |  |  |  |  |  |
| According to several studies, sorption of PAHs in sediment and soil and thus bioavailability seems to be dependent on the residence time but also on the sediment/soil type and exposure level. Adsorption of the 16 EPA-PAHs to organic carbon (expressed as log KOC) is between 3.13 (naphthalene) and 6.37 (indeno[1,2,3-cd]pyrene) according to the QSAR models. |  |  |  |  |  |
| 12.5 Results of PBT and vPvB assessment |  |  |  |  |  |
| The substance meets the criteria for PBT or vPvB |  |  |  |  |  |
| 12.6 Endocrine disrupting properties |  |  |  |  |  |
| The currently available information does not indicate that this substance has endocrine disrupting properties as defined by the criteria set out in Section B of Regulation (EU) No 2017/2100. |  |  |  |  |  |
| 12.7 Other adverse effects: |  |  |  |  |  |
| None |  |  |  |  |  |
| 12.8 Additional information |  |  |  |  |  |
| None. |  |  |  |  |  |

| 13. DISPOSAL CONSIDERATIONS |  |
| --- | --- |
| 13.1. Waste treatment methods |  |
| Appropriate disposal / Product | Dispose of waste and residues in accordance with local authority requirements. It can’t be disposed in hazardous waste landfills. |
| Waste codes / waste designations according to EWC / AVV | 05 06 03 (waste from pyrolysis of coal, other tars). |
| Appropriate disposal /Packaging | The generation of waste should be avoided or minimised wherever possible. Contaminated packaging should be disposed as the product’s waste. |

| 14. TRANSPORT INFORMATION |  |
| --- | --- |
| The product is transported by railway (RID), road (ADR) and the sea (IMDG) |  |
| 14.1 UN number | 3257 |
| 14.2 UN proper shipping name | ELEVATED TEMPERATURE LIQUID, N.O.S. (Coal tar pitch, high temp.) |
| 14.3 Transport hazard class(es) | 9 |
| 14.4. Packing group | III |
| 14.5. Environmental hazards | Marine polutant |
| 14.6. Special provisions | 274, 643, 668 EMS Number: F-A, S-P Limited quantities: 0 Excepted quantities: E0 |
| 14.7 Transport in bulk according to Annex II of MARPOL73/78 and the IBC Code | Not Applicable |
| 14.8. Additional information | Packaging instructions: P099, IBC99 |

15. REGULATORY INFORMATION

15.1 Safety, health and environmental regulations/legislation specific for the substance

Regulation (EC) No 1907/2006 Regulation (EC) No 1272/2008 This substance is subject for authorization (Entry no. 41)

15.2 Chemical Safety Assessment

No Chemical Safety Assessment has been carried out for this product by the supplier.

| 16. OTHER INFORMATION |  |
| --- | --- |
| Relevant H-, P- , EUH-phrases | H317: May cause an allergic skin reaction. H340: May cause genetic defects. H350 : May cause cancer. H360FD: May damage fertility. May damage the unborn child. H400: Very toxic to aquatic life. H410: Very toxic to aquatic life with long lasting effects. P201: Obtain special instructions before use. P202: Do not handle until all safety precautions have been read and understood. P280: Wear protective gloves/protective clothing/eye protection/face protection. P308 + P313: IF exposed or concerned: Get medical advice/attention. P273: Avoid release to the environment. P405: Store locked up P501: Dispose of contents and container in accordance with national or international regulations |
| Abbreviations | DNEL - The Derived No-Effect Level PNEC - Predicted no effect concentration PBT or vPvB - persistent, bioaccumulative and toxic or very persistent very bioaccumulative EC50, EC10– effect concentration LD50 - lethal dose LC50 - lethal concentration LL50 – loading rate NOAEL/NOEL - no observed adverse effect level NOEC - no observed effect concentration OEL – occupational exposure limit VLEP – valeurs limites d’exposition professionnelle - occupational exposure limit values VLA – valores límite ambientales- occupational exposure limit values VLE - valeurs limites d'exposition- occupational exposure limit values MAK, MAC - maximum workplace concentrations STOT SE – Specific target organ toxicity – single exposure STOT RE - Specific target organ toxicity – repeated exposure BCF – bioconcentration factor AF – Assessment factor |
| EU Poison centres contacts | AUSTRIA (Vienna Wien) +43 1 406 43 43; BELGIUM (Brussels Bruxelles) +32 70 245 245; BULGARIA (Sofia) +359 2 9154 409; CZECH REPUBLIC (Prague Praha) +420 224 919 293; DENMARK (Copenhagen) 82 12 12 12; ESTONIA (Tallinn) 112; FINLAND (Helsinki) +358 9 471 977; FRANCE (Paris) +33 1 40 0548 48; GERMANY (Berlin) +49 30 19240; GREECE (Athens Athinai) +30 10 779 3777; HUNGARY (Budapest) 06 80 20 11 99; ICELAND (Reykjavik) +354 525 111, +354 543 2222; IRELAND (Dublin) +353 1 8379964; ITALY (Rome) +3906 305 4343; LATVIA (Riga) +371 704 2468; LITHUANIA (Vilnius) +370 5 236 20 52 or +370 687 53378; MALTA (Valletta) 2425 0000; NETHERLANDS (Bilthoven) +31 30 274 88 88; NORWAY (Oslo) 22 591300; POLAND (Gdansk) +48 58301 65 16 or +48 58 349 2831; PORTUGAL (Lisbon Lisboa) 808 250 143; ROMANIA (Bucharest) +40 21 3183606 SLOVAKIA (Bratislava) +421 2 54 77 4166; SLOVENIA (Ljubljana) + 386 41 650500; SPAIN (Barcelona) +34 93 227 98 33 or +34 93 227 54 00 bleep 190; SWEDEN (Stockholm) 112 or +46 833 12 31 (mon-fri 9.00-17.00); UNITED KINGDOM (London) 112 or 0845 4647 (NHS Direct). |
| Training instructions | Read carefully the SDS before using the product. Train personnel in the safe use of the chemical substance. |
| Further information | The data contained in the safety data sheet is based on the amount of information and experience available to the company at this time. A consumer of product is responsible for the consequences of its use in specific purposes. Information refers to this particular product. It may be invalid in case this product is used together with any other materials or any other production process. |
| Key literature references and sources for data | Chemical safety report for Pitch, coal tar, high-temp. 2010-12-07 CSR-PI-5.2.6 European Union Risk Assessment Report for Coal-tar pitch, high temperature 2008 Annex XV transitional report for Coal-tar pitch, high temperature 2008 Summary risk assessment report (Environment) for Coal-tar pitch, high temperature 2008 ECHA database on registered substances GESTIS database on international limit values |

ANNEX 1

Exposure scenarios according to Chemical Safety Report

Not applicable, as the substance is manufactured and used only under strictly controlled conditions and is considered as transported isolated intermediate according to Article 3 (15).

The above data are in accordance with our knowledge and experience at the given date of revision and exclusively refer to the product in its as-delivered condition as it is unambiguously identifiable by the product number. In the case of usages deviating from those given in section 1 or when the product is mixed with other materials or is altered in the course of a production process, the statements given in the extended Safety Data Sheet may not apply without restrictions or even not at all any more. The data are not applicable to other products of the same or a similar designation.